- Cow dung supply: 350 cows ( Approximate cow dung: 350 x 15 kg = 5250 kg per day ).

- Digester technology: Continuous stirred Tank Reactor ( CSTR ) .

- Digester construction: fixed-type with reinforced concrete. Size 300 cubic meters. Insulation with thermophilic condition.

- Mixing System: Mechanical Stirrers.

- GAS storage: Double membrane roof.

- Electricity generator: 50 KW CHP generator ( 2 units) .

- Dewatering machine: Screw Pressed Separator.

- GAS purification system: Scrubber

- Dehumidifier